[image: image18.png]The

Pope Francis Catholic
Multi Academy Company






ASBESTOS MANAGEMENT PLAN
St Johns RC Primary School, Banbury
Date Approved:


13.08.2025
Next Review Date:

12/08/2026
Author/Reviewer:


Paul Stevenson/ Laura Houghton
CONTENTS

31.0
ASBESTOS MANAGEMENT POLICY STATEMENT


52.0
SCHOOL MANAGEMENT FLOWCHART


63.0
ASBESTOS MANAGEMENT PLAN


63.1
Duty Holder


63.2
Responsible Persons


63.3
Damaged ACMs / Emergency Procedure


73.4
Asbestos Survey and Register


103.5
Asbestos Containing Materials


123.6
Staff Awareness


123.7
Monitoring and Inspection


133.8
Works and Visitor Protocols


133.8.1
Contractors


143.8.2
Emergency Services


143.9
Refurbishment or Demolition Works


143.10
Asbestos Works and Removals


154.0
CONTROL MEASURES FOR SPECIFIC AREAS IDENTIFIED


16Appendix 1: Asbestos Containing Material Inspection Record


18Appendix 2: Contractor Review of Asbestos Register/Survey


20Appendix 3: Asbestos Permit to Work Proforma


20Permit to Work Issuing Instructions


22Appendix 4: Staff Training Records




1.0
ASBESTOS MANAGEMENT POLICY STATEMENT
St Johns RC has a local Asbestos Management Policy which can be found in the school office and the SchoolBus.
This Asbestos Management Plan sets out the actions taken within the school premises to manage Asbestos Containing Materials (ACMs) in accordance with The Control of Asbestos Regulations (CAR) and best practice guidance. 

A copy of this plan and the premises asbestos survey and register, in addition to any other relevant information will be held in the Asbestos folder in the school office.
This central folder will be made readily available to all those who need access to the asbestos documentation. 

This plan has been developed in consultation with details from the survey carried out by Amity and has been approved by the duty holder. 

To ensure school employees, pupils, contractors and visitors to the premises do not disturb ACMs and are safe from potential exposure, the following effective asbestos management procedures are in place:

· A designated person/s responsible for the management of asbestos on the schools premises (referred to as the Responsible Person); including the updating of existing records
· A system to ensure ACMs are identifiable through appropriate labelling and/ or colour coding
· Provision of asbestos awareness training to relevant school employees and third parties as deemed necessary (including the keeping of appropriate training records) 

· To periodically inspect ACMs on a regular basis as specified within the asbestos register

· To periodically review this Asbestos Management Plan

· Provide access to the asbestos management central folder to contractors carrying out maintenance and/ or construction works (this includes IT contractors) prior to the commencement of works
· Ensure that where deemed necessary, a refurbishment or demolition survey is undertaken when the school undertakes any construction works

· Seek advice and guidance from suitably qualified and experienced Competent Persons on any asbestos related work activities that are to be undertaken (this may include, but is not limited to: re-inspections, asbestos removal works, environmental cleans, encapsulation works & air-monitoring)  
· Inform Amity and the PFMAC Health and Safety services of any instances of suspected exposure to ACMs so that they can provide professional assistance and guidance (refer emergency procedures) 

2.0
SCHOOL MANAGEMENT FLOWCHART
[image: image18.png]
[image: image1]
3.0
ASBESTOS MANAGEMENT PLAN
3.1
Duty Holder

The school has appointed the Headteacher as the legally designated Duty Holder (unless an alternative person has been identified and appointed by the Governing Body).
3.2
Responsible Persons
The following members of staff have been nominated to be responsible for managing asbestos on the schools premises.

Site Manager / School Business Manager
The members of staff detailed above have attended a recent asbestos awareness training course. Details of these records can be found in the Asbestos Management Folder and in Appendix 4.
3.3
Damaged ACMs / Emergency Procedure

Where asbestos containing materials (ACMs) have been damaged, or damaged materials/suspected deterioration are identified during inspection processes the school will instigate the emergency procedure below. 

· Secure the area affected ensuring no access is permitted (signage should be displayed and barriers erected where appropriate)

· Review impact on the schools operational procedures i.e. if a classroom is affected, alternative teaching arrangements would need to be implemented as the classroom cannot be used
Contact the Headteacher and the Administration Manager, PFMAC Operations Manager and the Asbestos contractor Amity to notify them of the damage. 
Amity services will then provide professional advice and guidance as necessary. This may include, but is not limited to, inspecting the damage reported, arranging an air monitoring test and arranging and managing any associated remedial works required
· Maintain controlled access to the area until such time as formal clearance has been confirmed
· Maintain good communication with school staff and relevant other parties, providing updates as necessary to ensure the access arrangements are not breached

3.4
Asbestos Survey and Register

The Asbestos Survey provides accurate information on the location, amount and condition of ACMs. The information in the survey report will be used to form the asbestos register which is a key component of the management plan for the school. 

The school will ensure that an up-to-date copy of the asbestos survey/register for the will be available on the premises. 
This will be kept in the asbestos management folder.
In order to risk assess each ACM’s, we undertake 2 algorithms. 

The first is conducted during the asbestos management survey and given an overall material assessment score. 

The second is conducted by the responsible person and looks at the likelihood of people being exposed to the item and given an overall priority assessment score.

The 2 algorithms scores are then added together to give the overall risk score for each ACM. 

Material assessment score for each ACM

+

Priority assessment score for each ACM

=

Risk assessment score for each ACM 

All risk assessment scores will be listed on the asbestos register.
MATERIAL ASSESSMENT AND ALGORITHM

	Sample variable
	Score
	Examples of scores

	Product type (or debris   from product)
	1
	Asbestos   reinforced   composites   (plastic,   resins,   mastics, roofing felts, vinyl floor tiles, semi-rigid paints or  decorative finishes, asbestos cement)

	
	2
	Asbestos insulating board,  mill boards, other  low density insulation boards, asbestos textiles, gaskets, ropes  and  woven textiles, asbestos paper  and felt.

	
	3
	Thermal installation (i.e.  Pipe and   boiler lagging), sprayed asbestos, loose asbestos, asbestos mattresses and packing.

	Extent   of  damage

/ deterioration
	0
	Good condition; no visible damage

	
	1
	Low  damages:  a  few  scratches  or  surfaces  marks,   broken edges of tiles etc.

	
	2
	Medium damage: significant breakage of materials or several small areas where material has been damaged revealing loose asbestos fibres.

	
	3
	High   damage   or   delamination   of   materials,   sprays    and thermal insulation. Visible asbestos debris.

	Surface  treatment
	0
	Composite materials containing asbestos: reinforced plastics, resins, vinyl tiles.

	
	1
	Enclosed sprays  and  lagging, asbestos insulating board  (with exposed face painted or encapsulated) asbestos cement sheet etc.

	
	2
	Unsealed  asbestos insulating  board,  or  encapsulated  lagging and sprays

	
	3
	Unsealed laggings and sprays.

	Asbestos type
	1
	Chrysotile

	
	2
	Amphibole asbestos excluding crocidolite

	
	3
	Crocidolite

	No asbestos
	0
	No asbestos in sample

	Total score
	
	


PRIORITY ASSESSMENT ALGORITHM

	Assessment factor
	Score
	Examples of score variables

	Normal occupant activity
Main type of activity in area
Secondary activities of area
	
	

	
	0
	Rare   disturbance   activity   (i.e.   little   used   store room)

	
	1
	Low disturbance (i.e. office type activity)

	
	2
	Periodic disturbance (i.e.  industrial  or  vehicular activity which may contact ACM’s

	
	3
	High levels of disturbance (i.e.  fire  doors   with asbestos insulating board  sheet in constant use)

	
	As above
	As above

	Likelihood of disturbance
Location Accessibility Extent / amount
	
	

	
	0
	Outdoors

	
	1
	Large room or well ventilated areas

	
	2
	Room up to 100m²

	
	3
	Confined spaces

	
	0
	Usually inaccessible or unlikely to be disturbed

	
	1
	Occasionally likely to be disturbed

	
	2
	Easily disturbed

	
	3
	Routinely disturbed

	
	0
	Small amounts of items (i.e. strings, gaskets)

	
	1
	≤ 10m² or ≤ 10 m pipe run

	
	2
	≥ 10m² to ≤ 50 m2 or > 10m to ≤ 50 m pipe run

	
	3
	≥ 50m² or > 50 m pipe run

	Human exposure potential
Number  of occupants Frequency of use  of area Average time area  is in use
	
	

	
	0
	None

	
	1
	1 to 3

	
	2
	4 to 10

	
	3
	> 10

	
	0
	Infrequent

	
	1
	Monthly

	
	2
	Weekly

	
	3
	Daily

	
	0
	< 1 hour

	
	1
	> 1 to < 3

	
	2
	> 3 to < 6

	
	3
	> 6 hours

	Maintenance activity
	
	

	
	0
	Minor disturbance (i.e. possibility of contact when gaining access)

	
	1
	Low disturbance   (i.e.   changing   lights   bulbs   in asbestos insulating board  ceiling

	
	2
	Medium   disturbance   (i.e.   lifting   one    or   two asbestos insulating board  ceiling tiles to access  a valve)

	
	3
	High   levels   of   disturbance   (i.e.   removing   a number of asbestos insulating  board  ceiling  tiles to replace a valve or for re-cabling

	
	0
	ACM unlikely to be disturbed for maintenance

	
	1
	≤ 1 per year

	
	2
	> per year

	
	3
	> per month


3.5
Asbestos Containing Materials

The areas of the school which have asbestos containing materials (ACMs) that require management will be noted from the asbestos register within the asbestos survey report. A copy of the Asbestos Register is attached to this document. Controls for specific high risk areas identified are included later in this plan at Section 4.0 (Control measures for specific areas identified).

Where ACMs have been identified, the person/s named earlier in this plan as being responsible for managing asbestos will ensure that the materials are capable of being identified visually by all staff and contractors using the following: 

· Asbestos containing materials in classrooms, corridors and other areas accessible to all staff and pupils will be identified by a label/ sticker similar to those contained within the table below
· Asbestos containing materials in other areas will be labelled using labels commensurate with legislative requirements i.e. a ‘tombstone’ label (see table below)
	Examples of acceptable asbestos stickers/ labels that can used



	The following examples of labels/ stickers are suitable for use within the school premises as part of the management control procedures outlined within this Asbestos Management Plan. 

These examples are not extensive and other appropriate stickers/ labels may be used.

	Asbestos ‘tombstone’ sticker – 

normal industry standard label used

	
	[image: image19.emf] 

Contact the  Premises Manager  before  Working in  this  Area  



	Presumed asbestos sticker – 

used when similar materials have been proven to contain ACMs
	Encapsulated asbestos sticker – 

used when ACMs have been encapsulated

	[image: image20.png]Presumed
A asbestos
present




	[image: image2.jpg]Danger
A Encapsulated

asbestos






	Asbestos sticker – 

An alternative to the ‘tombstone’ sticker highlighted above
	Warning sticker  – 

Can be used in communal areas where ACMs are present; may be used in place of other types specified above which may cause unnecessary concern 

	[image: image21.png]




	[image: image22.png]Danger
asbestos







3.6
Staff Awareness 

Staff working within close proximity to asbestos or where located within area identified containing asbestos, will be provided with relevant information on:

I. Types and location of ACMs (via the Asbestos Register and Plan)

II. The visual means of identifying ACM’s (labels/colour coding)

III. How to avoid risks from asbestos (e.g. not disturbing)

IV. How to report concerns about ACMs (e.g. to the Duty Holder/responsible person) 
· Asbestos knowledge and awareness will be covered at any induction for new staff. Temporary staff should receive information if working in close proximity to areas identified containing asbestos.
· Staff can report any concerns in relation to ACM’s via contacting the administration manager directly or for non- urgent concerns or questions, via e-mail to the responsible person.
· Any changes to periodic updates on any asbestos related works will be communicated to staff via staff briefings, e-mail and any termly operations updates
3.7
Monitoring and Inspection

The school will ensure formal visual inspections of all known ACMs are carried our as stipulated within the Asbestos Register, recording the details of such inspections using a suitable proforma (refer Appendix 1 for template). 

In relation to each ACM score, The PFMAC has set some minimum standards based on the scoring process and the responsible person will follow the minimum requirements outline below per ACM identified and scored.
	ACM’s priority assessment score
	Minimum requirement

	25 to 20
	· Removal as priority

· Close off area and restrict all access

	19 to 15
	· Monitor 3 monthly

· Individual risk assessment reviewed annually or if change occurs

· Strong records keeping of checks and photos evidence

	14 to 10
	· Monitor 6 monthly

· Risk assessment for isolated ACM or areas containing more than one.
· Record keeping and photo evidence

	9 to 5
	· Monitor 12 monthly 
· Consider if any risk assessment is required

· Record keeping of checks

	4 to 0
	· Monitor 12 monthly 
· Record keeping of checks


Formal visual inspections of retained ACMs will be conducted on at least an annual basis, by a Competent Person qualified to BOHS P401 or P402 certificate level. All other periodical inspections stipulated will be undertaken by the Responsible Person/s identified earlier in this plan. These will be conducted and recorded as stipulated on the asbestos register.
Any damaged or deteriorated materials found will be reported according to the procedures detailed earlier in this plan at Section 3.3 (Damaged ACMs / Emergency Procedure).
The responsible person must ensure the annual external competent persons inspections are organised and carried as per management plan.
3.8
Works and Visitor Protocols

3.8.1
Contractors 
Everyone attending the school to carry out any works in a building or locations identified containing asbestos, will be required to access and review the asbestos survey, register and plan before undertaking any work. 

This will be provided by the person/s responsible for managing asbestos or other relevant member of staff within the school.

All contractors undertaking any work will be required to sign that they have reviewed the asbestos survey, register and plan.
Where there are ACMs that are to be worked on or nearby, no work will take place until an appropriate method statement of work is produced and the Permit-to-Work procedures detailed in Appendix 3 duly authorised and implemented. 

3.8.2
Emergency Services
Emergency Services personnel attending site must be given access to the asbestos survey, register and plan on arrival.

3.9
Refurbishment or Demolition Works

Where the school commissions any construction works involving an upgrade, refurbishment or demolition work a refurbishment or demolition survey will be undertaken as necessary to locate and describe, as far as is reasonably practicable, all ACMs in the area where the work will take place. 

This will be undertaken in accordance with the requirements of the Health and Safety Executive guidance HSG264.

Where necessary, the school will seek further advice and guidance from the PFMAC Health and Safety services or contacting the asbestos contractor Amity
3.10
Asbestos Works and Removals 

The school will ensure that any works undertaken involving ACMs will be carried out within the requirements of the Control of Asbestos Regulations 2012. Guidance from the Health and Safety Executive can be found at: http://www.hse.gov.uk/asbestos/index.htm. 
Control measures will be detailed in the method statement provided by the contractor. 

Where ACMs are to be removed or encapsulated etc, the duty holder, administration manager will be contacted prior to any such works taking place and provided with a copy of the contractor’s method statement etc. Confirmation should be received from the duty holder and school to proceed and all their records must be updated accordingly. 
The school asbestos register will be updated accordingly following completion of any asbestos related works.

Where the school requires further guidance in relation to the Control of Asbestos Regulations 2012, support from PFMAC Operations Manager and Amity 
4.0
CONTROL MEASURES FOR SPECIFIC AREAS IDENTIFIED
This section outlines specific control measures adopted by the school, which are outside of the normal adopted practices, for ACMs which staff, visitors, emergency services attending site and/or contractors must be made aware of.

There is a sink pad possibly containing encapsulated asbestos in the Staff Room. No control measures are required, beyond occasional inspection. 
Appendix 1: Asbestos Containing Material Inspection Record

	School
	St Johns RC, Banbury
	Date of Inspection
	See Amity Reports

	Inspection By (signature)
	Matthew Monro (Amity)
	Print Name
	


[image: image3.png]EE e e D T N in s - peariots o moren e
ool ol uiiurch et e s e
ETR [ P R [T I s I Crmeariors | Nosen o
il ol ety pre=iy P
I e e B e [ P i e T ey
ool e i oo prtrr [Seertey
S |G| ome oo s g [ [remwes |G [susones |G [ |G T ey
el e et | Pt et
P e P N e R I e e T ey
ol el ettt | feerr et
I o e o = [ o o o [Py T ey
ool b bt oo prtrr [Seertey
MR = e e T ey
o prtrr [oreey
MR = e e T ey
o prtrr [Seertey
EN e = B
pet by [Swori e
E TN C P e T ey
el [Seertey





[image: image4.png]Stk |Gound | ofe Gt womn w5 [ouss |Low |Gompeiy [Orpote [ |vsiyinaceine (3 [0-NoAnamion | s s nasnee
ool e oo gl PAcg Pty e [wioriesiil ok

S |G| o G |5 [mwos |G oy |G [em | vy (3 [0 heAmmien | Geem e
el e ey e | ot | S0 feerr [t e

e o T o e [ E T e Fe o e R [ = e
ool e berinchiaciey oo prtrr [Seertey

e o E e o e [ E T e P o e R [ = e
ool e ey oo prtrr [Seertey

I o] DT R e L E T e E o e R [ = e
ool e ey oo prtrr [Seertey

B e e e L N e O T
Pl bl et | b=t oo

i e e E L N A O T
ol bl et | b=t oo

I ] e o e L E T o P L Ty R [ = e
ool e o oo et [Seertey

e e L T o e [ E T o E L Ty R [ = e
ool e ity oo et [Seertey

e o L e o e L E T e L [ e R [ = e
ool e ey oo prtrr [Seertey





[image: image5.png]e e P R e = M =l o e ey o Aranion | e s

[t e et oo Pt [Seri Mol

ETr=R ) P R [T I s N Crmeariors [ Noeem e iy

Pl b e pre=iy P

e o P o = [ o o P ety T peeery

ool ey bbbl oo et [Seri Mol
= N

I ] T R ) [ e o o [y T ey

|| o e | cen prtrr [Seertey

P e e E N e R e R T ey

ol e et v e | e Pt oo

P e e N e R e T ey

Pl el el v e | e Pt oo

e ] T e E L o [ =i = ey T ey

Pl el et bed v e | e Pt oo

P e e N e R e e el T ey

Pl bl et v e | e Pt oo

P e e N e N e e T ey

Pl bl et v e | e Pt oo

I ] e o R [ E e o o [y T ey

ool el Keisoputtet ot el [Seertey

covig e





[image: image6.png]I e e e e e N T e -t Avarion [ e sing o g
el e kit v | e Pt [Seertey

e e L R e o e e T ey
ool e ety v | e Pt [Seertey

E e Ty ey R [ [ ey e B ey
ool e oy b e [Swori sy

S |G| e s |57 o senes T T ey
il el et feerr oo

Er e e B i e T ey
il el et fr=t oo

EE e ey Cam e T ey
ool e Kbyttt prerey [Seertey

EE e e Cam e T ey
ool e ptad prerey [Seertey

S |G s T e e v [T o
ool e pediartas prerey e[S
E e e e v [T o
ool e e prerey e[S
S| Goune | e et e g CEm T T ey
ool e o o [Seertey





[image: image7.png]oot ol bt e e it
D R e P L e e g ine
ol Pl bt S| et
B e e P L N e g ine
gl bl S| et
P g e P L I e g ine
Pl bl e S| et
FT gl PR eyt PRy [rowsriy gy Dy ey Py s F s s
ol bl it e, = e
F e e o L o P o E e =
Pl g e e e et
S [ s R e P e o o e I
Py nacccaig G sre = e et e e
ETr e Py T NG s W e s s s
p=g [t [t
- e o e
Pty
ET e Py I P [ Pl iy Py (e 1y s s
Py [y = e et oo o o s
" e pbibiie





[image: image8.png]=5
sl
o 7 = =l
[ EEE
b7 - .
- \ "3 3 o
. e o
o I e
S -] - | -
4 ol
. PO )
o ! —





Appendix 2: Contractor Review of Asbestos Register/Survey
This sheet should be signed by all those carrying out repair/maintenance work in a building or locations identified containing asbestos on the premises (including voluntary workers or staff) that will involve disturbing the fabric of the premises. 

Persons signing this sheet are signing to say that they have seen the Asbestos Register and Management Plan and checked that whether there is any known or presumed asbestos in the area in which they are working.

Where an asbestos containing material is suspected, no work should be carried out until all relevant procedures have been carried out as detailed in this plan.

[image: image9.png]HIGH RISK MATERIALS - SCORE

S 10+

Asbestos in poor condition, or 3sbestos debrisicontamination has been identified within the following aress isted in the table below. I i recommended that risk
assessment (5) are undertaken to ensure that Regulation 4, Regulation 10, Regulation 11, and Regulation 16 of the Controlof Asbestos Regulations 2012 are complied

with.

“There wers no resuts found

MEDIUM RISK MATERIALS - SCO
Asbestos containing materials, which are

RES 7-9

unsealed or damaged, have been

remedial work to seal o remove these materals i undertsken 35 3 prioriy

and that air monitoring s

identiied within the following aress listed

carrid out withi

in the table below. It s recommended that
in adjacent areas in order (o 2s5ess aitborne fibre

“There wers no resuts found





[image: image10.png]Building

Table 1 - Re-Inspection Report:

Table 1 - Re-inspection Report

Matorial

Condition

This
Inspoction
Matorial
Scoro

Surveyor Comments

[Rm———

2 | Stlohns School Printing G02 Textured coating | Textured Coating | Good Condition 3 Textured coating o celing D-No Attention
Required, Label
3 | stdohns School Offce G03 Textured coating | Textured Coating | Good Condition 3 Textured coating o caling D No Attenton
Required, Label
4| Stlohns School | Staffladies tolet GO4 | Textured coatng | Textured Coating | Good Condilon 3 Textured coating o caling D No Attention
Required, Label
5 | Stlohns School | Siaff gents tolet GO5 | Textured coatng | Textured Goating | Good Condion 3 Textured coating 1 caling D No Attention
Required, Label
& | Stohns School Store Go6. Textured coating | Textured Coating | Good Condition 3 Textured coating 1 caling D No Attention
Required, Label
7 | Stdohns School T room GO7 Textured coating | Textured Coating | Good Condition 3 Textured coating 1 caling D No Attention
Required, Label





[image: image11.png]Table 1 - Re-inspection Report

Building Matorial Condition This Surveyor Comments [R———
Inspoction
Matorial
Scoro

8 | Stlohns School T room GO7 Brown ties and Reinforced Low Damage 3 ‘Brown ties and adhesive under carpet | D - No Attention
‘adhesive. Composite Required, Label
0 | Stohns School Offce G0B Textured coating | Textured Coating | Good Condition 3 Textured coating t caling D No Attention
Required, Label
1| Stdohns School Offce G0B Brown ties and Reinforced Low Damage 3 ‘Brown ties and adhesive under carpet | D - No Attention
‘adhesive. Composite Required, Label
12 | Stdohns School Offce GOB ‘Cream ties and Reinforced | Good Condion 2 ‘Cream tles and adhesive under carpet | D -No Atiention
‘adhesive. Composite Required, Label
13 | Stohns School | Head teachers offce | Texlured coaing | Textured Coating | Good Condilon 3 Textured coating o caling D No Attention
c08 Required, Label
4 | Stdohns School | Staff oirouiation G10 | Textured coaing | Textured Goating | Good Condion 3 Textured coating 1 caling D No Attention
Required, Label
15 | Staohns School Entrance G11 Textured coating | Textured Coating | Good Condition 3 Textured coating 1 caling D No Attention
Required, Label
16 | Stohns School Cirouaton G12 | Textured coatng | Textured Goating | Good Condion 3 Textured coating 1 caling D No Attention
Required, Label





[image: image12.png]Table 1 - Re-inspection Report

Building Matorial Condition This Surveyor Comments [R———
Inspoction
Matorial
Scoro
17 | StJohns School | Reception classroom |  Textured coatng | Textured Coating | Good Condton 3 Textured coating o celing D-No Attention
G13 Required, Label
6 | Stdohns School Kichen G14 Textured coating | Textured Coating | Good Condition 3 Textured coating t caling D No Attention
Required, Label
19 | StJohns School | Reception classroom | Textured coaing | Textured Coating | Good Condiion 3 Textured coating t celing D No Attenon
2615 Required, Label
20 | Stdomns School | Disabled ket G16 | Textured coating | Textured Coating | Good Condition 3 Textured coating o caling D No Attenton
Required, Label
Stdonns School | School hall G18 | Textured coating | Textured Coating | Good Condiion 3 Textured coating o piched ceiing in hall | D - No Attention
Required, Label
‘Stdohns School | Servery orculation | Textured coating | Textured Coating | Good Condiion 3 Textured coating o piched ceiing in hall | D - No Attention
G2t Required, Label
25 | Stdohns School Gifs tollet G23 Textured coating | Textured Coating | Good Condition 3 Textured coating 1 caling D No Attention
+ Well Bound Required, Label
Material
29 | Stdohns School Boys tolel G24 | Textured coatng | Textured Goating | Good Condilon 3 Textured coating t caling D No Attention
+ Well Bound Required, Label
Materia





[image: image13.png]Table 1 - Re-inspection Report

Building Matorial Condition This Surveyor Comments [R———
Inspoction
Matorial
Scoro

31 | StiomsSchool | Year2classroom | Fextured coating | Textured Coating | Low Damage . Textured coating o celing B - Immediate
c2s + Well Bound Encapsulation

Material
34 | Stdohns School Yearsand 4 Textured coating | Textured oating | Good Condiion | 3. Textured coating t caling D No Attention
classroom G28 + Well Bound Required, Label

Material
35 | Stdomns School | Cloaks and crculation | _ Textured coating | Textured Coating | Good Conditon | 3 Textured coating t celing D No Attenon
G20 + Well Bound Required, Label

Materia
3 | Stdohns School “Gis tollet G30 Textured coating | Textured oating | Good Condiion | 3. Textured coating o caling D No Attention
+ Well Bound Required, Label

Materia
57 | Stdomns School | Year 12classroom | Textured coating | Textured Coating | Good Conditon | 3 Textured coating o celing D No Attention
3 + Well Bound Required, Label

Materia
38 | Stdomns School | SENDeofoom G32 | Texured coating | Textured Coating | Good Conditon | 3 Textured coating 1 caling D No Attention
+ Well Bound Required, Label

Materia
%0 | Stdohns School Ciroulaton G33 | Textured coaing | Textured Coating | Good Condiion | 3. Textured coating 1 caling D No Attention
+ Well Bound Required, Label

Material
41 | Stdohns School Cirouation G33 ‘Acoustc pads Reinforced Low Damage 3 “Acoustc pads to underside of sink unit | D -No Aliention
Composite. Required, Label





31-Year 2 Classroom G25 Removal of loose flaking artex coating from the affected area, attempt to refix plasterboard panel to framework using timber batten fixed through plasterboard , treat affected area with asbestos encapsulant and localised clean of area.

Completed 04.08.2025
[image: image14.png]Table 1 - Re-inspection Report

Building Matorial Condition This Surveyor Comments [R———
Inspoction
Matorial
Scoro
42 | StlomsSchool | BoysioletG34 | Texwred coating | Textured Coaing | Good Condifon | 3 Textured coating o celing D-No Attention
+ Well Bound Required, Label
Material

51 | StlomsSchool | CroualnG43 | Texired coating | Textured Coaing | Good Conditon | 3 Textured coating t caling D No Attention
Required, Label
52 | Stlomns School | Year classroom | Texured coating | Textured Coating | Good Conditon | 3 Textured coating o caling D~ No Attention
[ Required, Label
53 | Stdomns Schoal Lbrary 645 Textured coating | Textured Coating | Good Condiion | 3. Textured coating 1 celing D No Attention
Required, Label
54 | Stdomns Schoal St oom G46 Textured coating | Textured Coating | Good Condiion | 3| Textured coating o lower section of caing | D - No Attention
Required, Label
55 | Stdomns Schoal Cloaks G47 Textured coating | Textured Coating | Good Condiion | 3| Textured coating o lower section of caing | D - No Attention|
Required, Label
S5 | Stdomns Schoal Kichen G48 Textured coating | Textured Coating | _Good Condiion | 3. Textured coating t celing D No Attention
Required, Label
57 | Stdomns School | Breaastciub 49 | Texured coating | Textured Coating | Good Conditon | 3 Textured coating t caling D No Attention
Required, Label





[image: image15.png]Table 1 - Re-inspection Report

Building Matorial Condition This Surveyor Comments [R———
Inspoction
Matorial
Scoro

58 | StiomsSchool | Gascupboard GSO | Textured coating | Textured Coaing | Good Condiion 3 Textured coating o celing D-No Attention
Required, Label
59 | StJohns School Tollet G51 Textured coating | Textured Coating | Good Condition 3 Textured coating t caling D No Attention
Required, Label
50 | Stdohns School Store G52 Textured coating | Textured Coating | Good Condition 3 Textured coating t celing D No Attenon
Required, Label
51 | Stohns School Cupboard G53 Write tles and Reinforced Low Damage 3 “Write lor ties and adhesive in cupboard | D - No Attention
‘adhesive. Composite Required, Label
St Johns School Lobby G54, Textured coating | Textured Coating | Good Condition 3 Textured coating o caling D No Attention
Required, Label
Stdonns School | Extemal areas EO1 | Undercloaing | Asbesios Cement|  Low Damage ¥ Undercloaking o piched roof of garden | D - No Attention
store Required, Label
68 | Stdomns School | Extemalarcas 01 |  Undercloaking | Asbestos Cement| Low Damage. 4| Undercoaking to ptched roof of loak room. | D - No Attention
‘See draing for location Required, Label





[image: image16.png]PRESUMED ASBESTOS/NO ACCESS AREA

Asbestos Containing Materals have been presumed 25 being present o the following areas where access could not be gained. A management policy and plan needs o
identitythat these areas require inspection once access can be provided. These areas require re-inspection for accessibilty 3t § monthly ntervals

Buiding Floor Roomirea [—
St Jonns Sencol Root Voiz Boys oletlof space F02. £ - Inspect Prior to Disturbance
St Johns Sencol RootVois Year 12 cassroom ot space °03 E - inspect Pror o Disurbance
St Johns Sencol Growna Floor Seroinal 616 £ inspect Pror o Disurbance
St Johns Sencol sl el ez £01 E - inspect Pror o Disurbance






Appendix 3: Asbestos Permit to Work Proforma
Permit to Work Issuing Instructions

To be used when any work on/near asbestos containing materials is to take place


[image: image17]

Appendix 4: Staff Training Records

Staff’s training needs are subject to periodic review using the Smartlog training system
Periodic asbestos awareness training will be provided to those staff where the schools Training identifies the need. Other professional training will be undertaken as and when required.

These training records should be reviewed as a minimum annually and should form part of the staff’s Personal Development Plan and/or any Performance Reviews by the School Business Manager.
The Headteacher will complete Duty holder training via a suitable external provider and ensure this is kept up to date.

KEY: DARK BLUE TEXT      = SCHOOL ACTION


           DARK GREEN TEXT    = PFMAC or H+S ADVICE


           LIGHT BLUE TEXT       = CONTRACTOR ACTION (Amity)


 


 


 





Update Asbestos Survey & Register with Details 





COPY OF ASBESTOS SURVEY/ REGISTER ISSUED TO SCHOOL








NOMINATE PERSON/S TO BE RESPONSIBLE FOR MANAGING 


ASBESTOS CONTAINNG MATERIALS


(Provide them with learning and training) 





IDENTIFY ASBESTOS CONTAINING MATERIALS THAT REQUIRE MANAGEMENT ACTION (Asbestos Register will identify) 





LABEL 


& / OR


COLOUR CODE


ACM’S  





 DAMAGED ACM’S FOUND: EMERGENCY PLAN TAKES EFFECT





ISSUE PAPERWORK FOR SCHOOL ACTIVITY





SCHOOL CARRIES OUT ADDITIONAL


INSPECTION OF ACM’S


AS PER MGMT PLAN RQMTS





INFORMS STAFF/ CONTRACTORS OF ACM’S AND CONTROL MEASURES 





DEVELOP ASBESTOS MANAGEMENT PLAN 





ASBESTOS RELATED WORKS UNDERTAKEN





ACTIVITY TAKES PLACE (Under Controlled Conditions Where Applicable)





ARRANGE FOR REFURB/ DEMOLITION SURVEY IF APPLICABLE





CONTRACTOR ISSUES PAPERWORK





ACCREDITED COMPANY CARRIES OUT SURVEY & FORMAL REINSPECTIONS





PFMAC POLICY / H+S SCHOOLS GUIDANCE





SCHOOL LOCAL POLICY 


(if above not suitable)





EXTERNAL COMPETENT PERSON INSPECTS ACMs ANNUALLY





PROTOCOLS FOR MGMT OF CONTRACTORS / SCHOOL ACTIVITIES





EMERGENCY PLAN ARRANGEMENTS IN PLACE





BUILDING WORKS


DETERMINE WHETHER ANY OF THE FOLLOWING APPLY:-


1. Review Asbestos Register/ Plan


2. Refurb/ Demolition Survey Required


3. Risk Assessment & Method Statement Required


4. Permit to Work Required


5. HSE Notification (Asb5)








BUILDING WORKS


DETERMINE WHETHER ANY OF THE FOLLOWING APPLY:-


1. Review Asbestos Register/ Plan


2. Activity Risk Assessment & Method Statement Required


3. Permit to Work Required





SCHOOL ISSUES PAPERWORK





DETERMINE WHETHER ANY OF THE FOLLOWING APPLY:-


1. Inform departments /SLT/Governors


2. Review Asbestos Register/ Plan


2. Refurb/ Demolition Survey Required


3. Risk Assessment & Method Statement Required


4. Permit to Work Required


6. HSE Notification (Asb5)








DETERMINE WHETHER:-


1. HSE RIDDOR Notification Required


2. Incident Form Required


3. Notification to Staff, Pupils &/or Contractors Required





AMITY PROVIDES ADVICE & GUIDANCE





�





Section 4: Activity / Work NEAR asbestos containing materials


4.1 Has a method statement been prepared for the work?		       		Yes / No





The activity / works can commence once the above is confirmed but must stop immediately if any suspicious materials are discovered








ASBESTOS PERMIT TO WORK





Date/s for Works/ Activity:…………………………………………………………              Times:……………………………………





Section 1: Location and description of work





Section 2: Asbestos Register


2.1 Has the Asbestos Register been reviewed by the staff member/Contractor?   	Yes / No





2.2 Will the work disturb any asbestos containing material?				Yes / No





If the answer to 2.2 is YES – proceed to Section 3; if NO – proceed to Section 4





Section 3: Work ON asbestos containing materials


3.1 Contractors competency to work on ACM’s been confirmed?                       	Yes / No





3.2 Worked planned in accordance with the requirements of The Control


      of Asbestos Regulations 2012?				       			Yes / No





3.3 Duty holder + responsible person have been informed and given approval? 	Yes / No Work can commence once the above are confirmed





Section 5: School Approval





Signature: ………………………………………………………………………………..	Date:………………………………………………..





Print Name:…………………………………………………………………………….. (Responsible Person\








Section 6: Staff / Contractor Confirmation


I confirm that the activity / work will be undertaken in accordance with the requirements of this permit and school asbestos management plan.





Signature:………………………………………………………………………………..	Date:………………………………………………..





Print Name:………………………………………………… Company Name:…………………………………………………………........








 pg. 17

